NEW ENERGY  |  DATAOPS & AI INTERNSHIP

P08 — Day-Ahead Solar Generation Forecasting

Intern Reference & Project Guide

Contents

1.  Purpose & Business Context

2.  Learning Objectives

3.  Key Concepts You Should Understand First

4.  Framing the Forecasting Problem

5.  Data

6.  Step-by-Step Workflow

7.  Worked Example: Scoring One Day

8.  Common Pitfalls

9.  Tools & Environment

10.  Week-by-Week Plan

11.  Deliverables & Evaluation

12.  Resources & References

# 1. Purpose & Business Context

Day-ahead forecasting means predicting how much energy a solar plant will produce over the next day, before that day begins. A grid operator needs this number in advance to schedule conventional generation, balance supply and demand, and keep the grid stable. Because solar output swings with the weather, a good forecast is what lets a variable renewable plant behave like a dependable, schedulable resource.

In India this is not optional — it is regulated. Each day is divided into 96 time blocks of 15 minutes each, and a generator must declare a schedule for every block. The difference between what was scheduled and what was actually delivered is the deviation, and it is settled financially under the Deviation Settlement Mechanism (DSM). The more your actual output deviates from your declared schedule, the more you pay.

Why this matters commercially:  The Central Electricity Regulatory Commission (CERC) is phasing in tighter deviation norms from 1 April 2026 onward. Solar and wind-solar hybrid projects move toward a 10% deviation band, tightening to roughly 5% later in the phase-in, with further tightening targeted through 2031. A more accurate forecast directly reduces deviation penalties — so forecasting accuracy maps straight to rupees saved.

Business value of this project: a forecasting model that beats the naive baseline by even 15–20% can materially cut DSM penalties, improve scheduling decisions, and feed downstream energy-trading and storage-dispatch decisions. The intern is building a small but real piece of that capability.

# 2. Learning Objectives

By the end of this project the intern should be able to:

- Frame a real forecasting problem: define the target, horizon, and time resolution correctly.

- Assemble and align multi-source time-series data (plant generation + weather).

- Engineer meaningful features for solar forecasting (time, lag, and weather-derived features).

- Build and compare baseline, tree-based, and sequence (deep-learning) models.

- Evaluate forecasts honestly using the right metrics and a proper time-series validation scheme.

- Communicate results: quantify the uplift over a naive baseline and explain the model's weaknesses.

# 3. Key Concepts You Should Understand First

Solar generation is driven mostly by how much sunlight hits the panels. A handful of concepts explain almost all of the variation — learn these before touching the data.

| Term | What it means / why it matters |
| --- | --- |
| GHI / DNI / DHI | Components of solar irradiance (W/m²). GHI = Global Horizontal Irradiance (total on a flat surface) — the single most important driver of output. |
| POA irradiance | Plane-of-Array irradiance: sunlight actually hitting the tilted panel. Closer to what the plant 'sees' than GHI. |
| Clear-sky irradiance | The theoretical irradiance on a perfectly cloudless day, computed from the sun's geometry (location, date, time). Deterministic and known in advance. |
| Clear-sky index (kt) | kt = measured GHI ÷ clear-sky GHI. It strips out the predictable sun-geometry pattern and isolates the weather effect (clouds). A very powerful feature. |
| Capacity factor (CF) | Actual energy produced ÷ energy if running at full rated capacity. Useful for sanity-checking outputs. |
| Performance Ratio (PR) | How efficiently the plant converts available irradiance into energy, after losses (heat, soiling, wiring). |
| Night-time zeros | Output is exactly 0 at night. Roughly half of all 15-min blocks are zero — this skews error metrics if you are not careful (see §6.6). |

The key insight for day-ahead:  Because you forecast before the day starts, you cannot use the actual weather of that day — you only have a weather forecast (a Numerical Weather Prediction, or NWP). Day-ahead solar accuracy is therefore capped by weather-forecast accuracy. Build your pipeline around forecasted weather inputs, not measured ones (a classic mistake — see Pitfalls).

# 4. Framing the Forecasting Problem

| Design choice | Recommended for this project |
| --- | --- |
| Target variable | Plant energy generation (MWh) — or average power (MW) — per time block. |
| Forecast horizon | Next calendar day: blocks for the following 24 hours (extendable to 48h). |
| Time resolution | Start with hourly to keep it simple, then move to 15-min (96 blocks/day) to match Indian scheduling. |
| Inputs (features) | Forecasted weather (irradiance, temperature, cloud cover, humidity, wind) + calendar/solar-geometry features + recent lags. |
| Problem type | Supervised regression on tabular features (XGBoost) and/or sequence model (LSTM). Prophet as a quick time-series benchmark. |

A useful mental model: you are learning a function that maps tomorrow's predicted weather and the time of day to tomorrow's expected generation, block by block.

# 5. Data

## 5.1 What you need

| Data | Examples | Where to get it (for a prototype) |
| --- | --- | --- |
| Plant generation history | MWh or MW per 15-min/hourly block | Plant SCADA exports (internal); for prototyping use public PV plant datasets |
| Historical & forecast weather | GHI, temp, cloud cover, humidity, wind | Open-Meteo API, NREL NSRDB, PVGIS (free, well-documented) |
| Plant metadata | Rated capacity (MW), tilt, location (lat/long) | Plant nameplate / commissioning documents |

Start public, then swap in internal:  Build the entire pipeline on a free public dataset and a free weather API first. Only after access is approved do you point the same code at internal SCADA data. This avoids being blocked on data-access paperwork in week 1.

## 5.2 Example data schema

After joining generation and weather on the timestamp, your training table should look like this:

| timestamp | gen_mwh | ghi | temp_c | cloud_% | wind_ms |
| --- | --- | --- | --- | --- | --- |
| 2026-05-01 08:00 | 1.52 | 210 | 27.4 | 15 | 2.1 |
| 2026-05-01 09:00 | 3.81 | 455 | 29.1 | 10 | 2.4 |
| 2026-05-01 10:00 | 6.20 | 690 | 31.8 | 20 | 3.0 |
| 2026-05-01 11:00 | 8.47 | 835 | 33.5 | 35 | 3.3 |
| 2026-05-01 12:00 | 9.10 | 905 | 34.9 | 40 | 3.6 |
| 2026-05-01 13:00 | 8.83 | 870 | 35.2 | 30 | 3.4 |

Notice generation tracks GHI closely, dips when cloud cover rises (block 11:00 vs 12:00), and is bounded by the plant's rated capacity (10 MW in this example).

# 6. Step-by-Step Workflow

## 6.1 Explore the data (EDA)

Plot generation against time and against GHI. Things you should expect to see and confirm:

- A clear daily bell-shaped curve, zero at night, peaking near solar noon.

- A strong, near-linear relationship between GHI and generation.

- Cloudy days producing jagged, suppressed curves vs smooth clear-sky days.

- Seasonal differences (longer, higher curves in summer).

## 6.2 Feature engineering (with examples)

This is where most of the accuracy is won. Three families of features:

a) Calendar / solar-geometry features — hour of day, month, day-of-year, and ideally solar elevation angle. These capture the deterministic sun pattern.

b) Weather features — forecasted GHI/POA, temperature, cloud cover, humidity. GHI is usually the strongest single predictor.

c) Derived / lag features — clear-sky index, and recent generation lags where the use-case allows. For pure day-ahead, prefer same-block-yesterday rather than last-hour (which you would not have).

import pandas as pd, numpy as np

df["ts"] = pd.to_datetime(df["timestamp"])

df["hour"]   = df["ts"].dt.hour

df["month"]  = df["ts"].dt.month

df["doy"]    = df["ts"].dt.dayofyear

# cyclical encoding so 23:00 and 00:00 are "close"

df["hour_sin"] = np.sin(2*np.pi*df["hour"]/24)

df["hour_cos"] = np.cos(2*np.pi*df["hour"]/24)

# clear-sky index (needs a clear-sky GHI column, e.g. from pvlib)

df["kt"] = (df["ghi"] / df["ghi_clearsky"]).clip(0, 1.2).fillna(0)

# same block yesterday (safe for day-ahead)

df["gen_yesterday"] = df["gen_mwh"].shift(24)   # 24 = blocks/day at hourly res

## 6.3 Always build a baseline first

You cannot claim a model is good without something to beat. Two standard baselines:

- Persistence: "tomorrow's block = same block yesterday." Trivial, but surprisingly hard to beat on calm days.

- Clear-sky persistence: assume tomorrow's clear-sky index equals today's, then scale by tomorrow's clear-sky GHI. A stronger, solar-aware baseline.

Rule of thumb:  If your fancy model cannot beat persistence, something is wrong with your features or validation — not with persistence.

## 6.4 Candidate models

| Model | Why use it | Notes |
| --- | --- | --- |
| XGBoost / LightGBM | Strong on tabular weather features; fast; gives feature importance. | Recommended primary model. |
| Prophet | Quick seasonal/trend benchmark with little tuning. | Good sanity-check, weak on weather-driven spikes. |
| LSTM / GRU | Captures temporal sequence patterns directly. | Stretch goal; needs more data and care. |
| Linear regression | Interpretable floor model. | Use as a second baseline. |

from xgboost import XGBRegressor

features = ["hour_sin","hour_cos","month","ghi","temp_c","cloud_%","kt","gen_yesterday"]

model = XGBRegressor(n_estimators=400, max_depth=6,

learning_rate=0.05, subsample=0.8)

model.fit(X_train[features], y_train)

pred = model.predict(X_test[features])

pred = np.clip(pred, 0, PLANT_CAPACITY_MWH)   # never predict <0 or >capacity

## 6.5 Validate the right way (time-series split)

Time-series data must NEVER be shuffled. The future cannot be used to predict the past. Use a walk-forward (expanding-window) split: train on earlier dates, test on later dates, then roll forward.

Fold 1:  train [Jan–Mar]            -> test [Apr]

Fold 2:  train [Jan–Apr]            -> test [May]

Fold 3:  train [Jan–May]            -> test [Jun]

(always test on dates AFTER the training window)

In scikit-learn use , not the default .

## 6.6 Evaluation metrics

Use several metrics — each tells you something different.

| Metric | Formula (plain text) | Reading it |
| --- | --- | --- |
| MAE | mean( ¦forecast − actual¦ ) | Average error in MWh. Easy to explain. |
| RMSE | sqrt( mean( (forecast − actual)² ) ) | Penalises large misses more. Main tuning metric. |
| nRMSE | RMSE ÷ plant capacity | RMSE as a % of capacity — lets you compare plants. |
| MBE (bias) | mean( forecast − actual ) | Sign matters: + = over-forecast, − = under-forecast. |
| Skill score | 1 − (RMSE_model ÷ RMSE_baseline) | Fraction better than persistence. This is the headline number. |

Beware MAPE at night:  MAPE divides by the actual value, and solar output is 0 at night — so MAPE explodes (divide-by-zero). Either exclude night-time blocks, or normalise by capacity (nRMSE) instead. This is the single most common metric mistake in solar forecasting.

# 7. Worked Example: Scoring One Day

A small, fully worked example so the metrics are concrete. Consider a 10 MWh-capacity plant, six daylight hours, with your model's forecast, the actual generation, and a persistence baseline (yesterday's actuals).

| Hour | Actual | Forecast | Persist. | Error (F−A) | Error² |
| --- | --- | --- | --- | --- | --- |
| 08:00 | 1.5 | 1.2 | 2.0 | −0.3 | 0.09 |
| 09:00 | 3.8 | 4.1 | 3.0 | +0.3 | 0.09 |
| 10:00 | 6.2 | 5.8 | 5.5 | −0.4 | 0.16 |
| 11:00 | 8.5 | 7.9 | 9.0 | −0.6 | 0.36 |
| 12:00 | 9.1 | 9.4 | 8.5 | +0.3 | 0.09 |
| 13:00 | 8.8 | 9.0 | 8.0 | +0.2 | 0.04 |
| Sum | — | — | — | −0.5 | 0.83 |

All values in MWh. Working through the metrics for the model:

Skill score vs persistence: the persistence forecast gives an RMSE of 0.66 MWh (sum of its squared errors = 2.63, ÷6 = 0.438, sqrt = 0.66). So:

Skill = 1 − (RMSE_model / RMSE_persistence)

= 1 − (0.37 / 0.66)

= 0.44   ->  the model is ~44% better than persistence

Interpretation: a skill score of 0.44 is a strong result for day-ahead solar. The project success metric ("beat persistence by >20%", i.e. skill > 0.20) would be comfortably met here.

# 8. Common Pitfalls (read before you start)

- Data leakage: Using the day's actual weather (or actual generation) as a feature. Day-ahead means only forecasted weather and past data are allowed.

- Shuffling time series: Random train/test splits let the model peek at the future. Always split chronologically.

- MAPE blow-up: Dividing by zero night-time output. Exclude night blocks or normalise by capacity.

- Forgetting physical limits: Clip predictions to [0, capacity]. A model predicting −2 MW or 14 MW on a 10 MW plant is wrong by construction.

- Timezone / DST errors: Mismatched timestamps between SCADA and weather data silently destroy accuracy. Align to one timezone (IST) up front.

- No baseline: Reporting RMSE with nothing to compare it to. Always report the skill score vs persistence.

# 9. Tools & Environment

Language: Python 3. Core libraries: pandas, numpy, scikit-learn, xgboost/lightgbm, matplotlib/plotly. Solar-specific: pvlib (clear-sky models, solar geometry). Optional: prophet, pytorch (LSTM), streamlit (demo dashboard).

python -m venv venv && source venv/bin/activate

pip install pandas numpy scikit-learn xgboost matplotlib pvlib prophet

# weather data: Open-Meteo (free, no key) or NREL NSRDB / PVGIS

# 10. Execution Plan

Focus & deliverable

Set up environment. Assemble generation + weather data, align timestamps to IST, handle gaps. Produce an EDA notebook with the plots from §6.1.

Feature engineering (§6.2). Build the persistence and clear-sky-persistence baselines and record their RMSE — this is your bar to beat.

Train and tune XGBoost/Prophet. Run an initial walk-forward backtest. First skill-score number.

Add an LSTM (stretch) and/or refine features. Full walk-forward backtesting across multiple folds.

Tune and compare all models. Error analysis: when/why does it fail (cloudy days, mornings)? Quantify uplift vs baseline.

Package: clean repo + README, final report, short slide deck, and a live/recorded demo. Optional: P10/P50/P90 probabilistic intervals.

# 11. Deliverables & Evaluation

Expected deliverables:

- A clean, reproducible Git repository with a README and requirements file.

- An EDA notebook and a modelling notebook/script.

- A backtest report comparing all models against the baselines, with the skill score as the headline.

- A short slide deck and a 10-minute demo.

Success metric: the model's day-ahead nRMSE/MAPE beats the persistence baseline by more than 20% (skill score > 0.20), validated on a proper walk-forward backtest.

Evaluation weighting:

| Dimension | Weight |
| --- | --- |
| Problem understanding & framing | 20% |
| Technical execution (features, models, validation) | 30% |
| Result quality vs success metric | 25% |
| Communication & documentation | 15% |
| Initiative / stretch goals | 10% |

# 12. Resources & References

Data & tooling:

- Open-Meteo — free historical & forecast weather API (no API key).

- NREL NSRDB — National Solar Radiation Database (irradiance).

- PVGIS (EU) — solar resource and PV performance data.

- pvlib-python — clear-sky models, solar position, PV modelling.

Regulatory context (India scheduling & DSM):

- CERC phased DSM reform for wind & solar — tightening deviation norms (phase-in from April 2026 through 2031).

- Scheduling in 96 fifteen-minute time blocks per day; deviation settled per block at the pooling-station level.

Sources consulted for the regulatory context:

SolarQuarter — "CERC Announces Phased DSM Reform For Wind And Solar, Tightens Deviation Norms By 2031."

Mercom India — "Better Forecasting Tools Vital to Prevent Deviations by Renewable Energy Generators."

CERC (Deviation Settlement and Related Matters) Regulations, 2024 [Draft].